# RISC V MMU SoC Architecture

Brief design description for RTL verification FPGA and ASIC implementation

# System Overview

The design is a single-cycle 32-bit RV32I system-on-chip with separate instruction and data translation paths. Each path uses a small TLB backed by a configurable page table. APB registers control MMU enable, page-table entries, TLB flush, fault clearing and UART transmission. Fault logic blocks unsafe writes, substitutes a NOP on instruction faults, and records sticky diagnostics.

# Top Level Data Flow

| Stage | Main blocks | Function |
| --- | --- | --- |
| Execute | Single_Cycle_RV32I, control_logic, datapath, regfile, alu32 | Fetch, decode and execute RV32I instructions; form data addresses and stores |
| Translate | instruction_mmu, data_mmu, tlb ×2, page_table | Translate 32-bit virtual addresses; enforce R/W/X permissions; report hit, miss and fault |
| Memory | imem, dmem, two SRAM macros in ASIC | 4 KiB instruction and 4 KiB data spaces; data writes are fault-gated |
| Control | apb_mmu_control | Configure MMU, PTBR/PTE state, flush TLBs, clear faults and drive UART |
| Observe | fault_handler, gpio_debug, UART and debug ports | Sticky fault capture, status visibility and serial/GPIO output |

# Address Translation Behavior

When MMU enable is low, virtual addresses pass through as physical addresses.

With MMU enabled, the TLB matches the virtual page number and supplies the physical page number and permission flags.

A miss obtains the mapping from the page table and refills the corresponding instruction or data TLB.

Invalid or disallowed instruction accesses return a NOP to the CPU; invalid or disallowed data writes are suppressed.

Page offset bits are preserved through translation.

# Memory and Control Architecture

Instruction and data traffic are Harvard-style at the SoC boundary. The CPU continuously presents the program counter to the instruction MMU. Data translation becomes valid for loads and stores, with write intent derived from the CPU store control. The physical-design configuration hardens the data memory as two 2 KiB 1RW1R SRAM macros. The SoC_Hardened wrapper ties APB inputs inactive and exposes only clock, reset, UART and four GPIO bits to keep the ASIC pin count practical; the full APB and debug interfaces remain in Top for verification and FPGA use.

# Reset Fault and Safety Behavior

CPU reset is asserted by external reset or active data-memory clearing.

Instruction faults cause NOP injection, preventing execution of an invalid fetched word.

Data page or permission faults gate the memory write enable.

Fault handler retains sticky instruction/data page and permission faults plus a fault address until cleared.

TLB flush and fault-clear controls are software-visible through APB.

# Verification Interfaces

The RTL exports virtual/physical addresses, access-valid/write qualifiers, TLB hit/miss signals, immediate and sticky faults, MMU enable, UART busy and data-write commit. UVM environments monitor APB and VM status, compare observed behavior in a scoreboard, sample functional coverage, and bind protocol/MMU assertions.

# Implementation Variants

| Variant | Clock intent | External interface | Memory realization |
| --- | --- | --- | --- |
| Verification Top | Testbench controlled | Full APB plus debug visibility | Behavioral RTL memories |
| FPGA fpga_top | Board clock constrained by arty.xdc | Board UART GPIO/reset/clock | Synthesized fabric in supplied build |
| ASIC SoC_Hardened | 100 MHz; 10 ns | clk, reset, UART, GPIO[3:0], power pins | Two 2 KiB SKY130 SRAM macros plus standard cells |