Synthesis Report

RISC-V RV32I SoC with MMU, APB, UART and GPIO

# Executive Summary

The supplied ASIC synthesis run maps the hardened SoC to the SKY130 high-density standard-cell library and preserves two SRAM macros. Yosys reports 916 mapped cells and 7,912.59 µm² of known standard-cell area; the SRAM macro area is explicitly reported as unknown in the synthesis statistic, so this is not the full block area. No latches were inferred and the synthesis flow reached a mapped netlist without unmapped cells.

| Metric | Result | Interpretation |
| --- | --- | --- |
| Top module | SoC_Hardened | ASIC hardened wrapper used by the physical-design flow |
| Mapped cells | 916 | Includes two SRAM macro instances |
| Known cell area | 7,912.5888 µm² | Excludes unknown SRAM macro area in Yosys stat |
| Sequential known-cell area | 2,825.2096 µm² (35.71%) | Flip-flop contribution to known mapped-cell area |
| SRAM macros | 2 × sky130_sram_2kbyte_1rw1r_32x512_8 | Macro area not included in Yosys area total |
| Unmapped cells | 0 | Mapped netlist is complete |
| Inferred latches | 0 | Combinational processes synthesize without latch inference |

# Cell Composition

| Cell type | Count |
| --- | --- |
| sky130_fd_sc_hd__dfxtp_2 | 118 |
| sky130_fd_sc_hd__dfrtp_2 | 11 |
| sky130_fd_sc_hd__dfstp_2 | 1 |
| sky130_fd_sc_hd__buf_2 | 328 |
| sky130_fd_sc_hd__and2_2 | 87 |
| sky130_fd_sc_hd__conb_1 | 93 |
| sky130_fd_sc_hd__mux4_2 | 7 |
| sky130_sram_2kbyte_1rw1r_32x512_8 | 2 |

# FPGA Synthesis Cross-Check

The FPGA branch contains a completed F4PGA/Yosys build for an Artix-7 Arty target. Its final mapped top-level statistic provides a useful independent synthesis sanity check, although it is not directly comparable to SKY130 ASIC area.

| FPGA primitive/resource | Count |
| --- | --- |
| Total mapped cells | 285 |
| LUTs | 122 |
| CARRY4_VPR | 9 |
| FDCE_ZINI | 47 |
| FDPE_ZINI | 9 |
| MUXF6 | 41 |
| Input buffers | 4 |
| Output tri-state buffers | 5 |

# Synthesis Quality Assessment

PASS: design synthesized to technology cells with no unmapped cells.

PASS: no unintended latches are reported in the supplied synthesis latch report.

PASS: memory hardening is reflected by two SRAM macro instances rather than a large synthesized register array.

## Source Traceability

Primary evidence used for this report:

riscv_mmu_pd/runs/RUN_2026-09-13_10-44-24/06-yosys-synthesis/reports/stat.rpt

riscv_mmu_pd/runs/RUN_2026-09-13_10-44-24/06-yosys-synthesis/reports/latch.rpt

riscv_mmu_pd/runs/RUN_2026-09-13_10-44-24/final/metrics.json

fpga/riscv_mmu/build/arty_100/fpga_top_synth.log